Data Pack — README

This pack contains everything referenced in the assignment: two telemetry files, two matching price files, and the compliance rulebook. All files are synthetic but built to behave like real SCADA/market data, including realistic data quality problems. Nothing here needs external lookups.

# Files

| File | Use | Rows | Period |
| --- | --- | --- | --- |
| turbine_telemetry_14day_sample.csv | Part A (agent) | 1,680 | 2026-03-01 to 03-14 |
| dam_price_14day_sample.csv | Part A (agent) | 336 | 2026-03-01 to 03-14 |
| turbine_telemetry_90day.csv | Part B (forecast) | 10,725 | 2026-03-01 to 05-29 |
| dam_price_90day.csv | Part B (forecast) | 2,160 | 2026-03-01 to 05-29 |
| compliance_rulebook.docx | Part A (agent) | — | reference document |

The 14-day files are an exact slice of the first 14 days of the 90-day files — same generation process, same turbine fleet, same units. Use the 14-day files for Part A so the agent has less data to reason over; use the full 90-day files for Part B where you need enough history to forecast.

# turbine_telemetry_*.csv

| Column | Type | Description |
| --- | --- | --- |
| timestamp | datetime, hourly | One row per turbine per hour |
| turbine_id | string | T01–T05, five turbines in one wind farm |
| wind_speed | float | Hub-height wind speed, expected unit m/s |
| power_kw | float | Active power output, kW. Nameplate capacity is 2,000 kW per turbine |
| availability | float (0 or 1) | 1 = turbine online and reporting, 0 = known offline |

- Not every turbine has a row for every hour. Missing rows mean missing telemetry, not zero output — don't assume a gap equals zero.

- The power curve is a standard cubic ramp between cut-in (3 m/s) and rated speed (12 m/s), flat at rated capacity up to cut-out (25 m/s), plus sensor noise. If a turbine's power doesn't track its reported wind speed the way you'd expect from that curve, that's worth investigating rather than averaging over.

- This is real-world-shaped data, not clean textbook data. We are not going to tell you what's wrong with it — finding and handling that is part of the assignment.

# dam_price_*.csv

| Column | Type | Description |
| --- | --- | --- |
| timestamp | datetime, hourly | Same clock as the telemetry files |
| dam_price_inr_per_kwh | float | Day-ahead market clearing price, ₹/kWh |

Price has a diurnal shape (evening peak, overnight trough), a weekday/weekend effect, and occasional demand spikes. It is not turbine-specific — it's one series for the whole period, representing the grid price the farm sells into or the electrolyzer buys from.

# compliance_rulebook.docx

A short, self-contained set of six rules governing RFNBO temporal correlation, curtailment handling, and reporting granularity. It's deliberately brief — if a question can't be answered from these six rules, the correct agent behavior is to say so, not to guess.

# What we won't tell you

The telemetry data has known, deliberate quality issues — the kind you'd actually hit with SCADA data (comms outages, sensor calibration drift, a unit mismatch after a firmware update, that sort of thing). Part of Part B is finding and documenting them yourself. Write up what you find and how you handled it; that's more valuable to us than a model that silently trains through bad data.